# How to calculate the slide parameter for OsxAptioFix2Drv

If you cannot boot because of an error like "couldn't allocate runtime area, error allocating 0xXXXX pages (...)" with OSXAptioFIX2 then your memory is too fragmented. You need to manually "slide" the pointer of the OSXAptioFix2 to a free continuous map of at least the size of 0xXXXX in your memory. Because this memory map is unique to each system configuration you have to use a different slide command on most computers.

Step 1.  Get memory map

Boot into the UEFI Shell from Clover and enter the command memmap. You'll get an output like similar to this:

Step 2. Open calculator

Open the calculator and choose the option View -> Programmer. Switch the calculator to hex mode by pressing the button labelled 16. It’s one of the buttons labelled 8|10|16.

Step 3. Find the large enough block

Look at the column # Pages (N Pages) for the number bigger than the one You got in the error message. The Type column should also indicate that this is Available block. For example if You got 0x146BD then 1DA8F is available bigger than this requested block and You can use the Start column from Your memmap command.

Once You find a Start calculate the slide like this

(Start – 100000) / 200000    or for this example   (6B78000 – 100000)  / 200000

Step 4. Convert to base 10

Now just switch Your calculator to base 10 by pressing the number labelled 10.

In the example the slide is 53.